Fedora + Windows VM Ultimate Steward Guide (v5 — Full)

TRX40 AORUS MASTER (rev 1.0) • Threadripper 3960X • GTX 1080 Host • RTX 3090 VM • Elastic CPU + Elastic RAM

# Quick Start — Golden Path Checklist (baseline path)

Follow this once. If you deviate, return to the last checkpoint that was known-good.

Checkpoint 1 — BIOS done

Expected result: Fedora boots cleanly and the display is driven by the GTX 1080 (host GPU).

Checkpoint 2 — Virtualization stack installed

Expected result: virt-manager opens without errors and libvirt is running.

Checkpoint 3 — IOMMU groups look sane

Expected result: the RTX 3090 GPU + its audio function are grouped reasonably, without dragging unrelated controllers with it.

Checkpoint 4 — VFIO binding complete

Expected result: host shows “Kernel driver in use: vfio-pci” for the RTX 3090 functions.

Checkpoint 5 — VM created (before Windows install)

Expected result: VM uses OVMF (UEFI), Q35, VirtIO disk+NIC, and has the RTX 3090 passed through (VGA + Audio).

Checkpoint 6 — Windows installed + drivers

Expected result: VirtIO drivers installed, NVIDIA driver installed, stable boot to desktop.

Checkpoint 7 — Snapshot baseline

Expected result: a named snapshot exists before any hardening/updates (see Section 12).

# 0. What this system is

Fedora is your real operating system. Windows lives inside a VM and only runs when you start it. The RTX 3090 is passed through to Windows for near-native GPU performance. The GTX 1080 stays with Fedora so the host remains stable, responsive, and in control.

# 1. The Steward Philosophy (read once, then follow the steps)

Conservative first: build a baseline that always boots and always recovers.

Elastic CPU is default: Linux schedules CPU time dynamically; no pinning needed.

Elastic RAM is explicit: use virtio ballooning so Windows can grow while Fedora stays healthy.

No guesswork: change one thing at a time; snapshot before risk.

# 2. Target Hardware + Layout

CPU: AMD Threadripper 3960X (24C/48T)

RAM: 128 GB DDR4

Motherboard: Gigabyte TRX40 AORUS MASTER (rev 1.0)

Host GPU (Fedora): GTX 1080

VM GPU (Windows): RTX 3090

Recommended physical layout (simple and stable):

GTX 1080 in the top x16 slot (host display GPU).

RTX 3090 in the next full-length x16 slot (passthrough GPU).

Monitor 1 plugged into GTX 1080 (Fedora).

Monitor 2 or a second input plugged into RTX 3090 (Windows VM).

# 3. BIOS setup — exact menu paths (TRX40 AORUS MASTER)

Enter BIOS: press DEL during boot. Press F2 to switch to Advanced Mode. Change only these settings:

## 3.1 Enable CPU virtualization

Path: Advanced Mode → Tweaker → Advanced CPU Settings

SVM Mode → Enabled

## 3.2 Enable IOMMU

Path: Advanced Mode → Settings → IO Ports

IOMMU → Enabled

## 3.3 PCIe addressing and memory mapping

Path: Advanced Mode → Settings → IO Ports

Above 4G Decoding → Enabled

Re-Size BAR Support → Disabled (keep disabled for baseline stability)

## 3.4 Force host GPU as boot display

Path: Advanced Mode → Settings → IO Ports

Initial Display Output → the PCIe slot containing the GTX 1080

## 3.5 UEFI-only boot

Path: Advanced Mode → Boot

CSM Support → Disabled

## 3.6 Disable Secure Boot (baseline)

Path: Advanced Mode → Boot → Secure Boot

Secure Boot Enable → Disabled

## 3.7 Save and exit

Path: Advanced Mode → Save & Exit

Save & Exit Setup → Confirm, then reboot once before installing Fedora

# 4. Install Fedora (host OS)

Install Fedora Workstation (or Fedora KDE if you prefer a more traditional desktop).

During install: consider full-disk encryption if you want maximum privacy at rest.

After first boot, update the system and reboot.

sudo dnf upgrade -y

sudo reboot

# 5. Install virtualization stack (KVM/QEMU/libvirt/virt-manager)

Install the Fedora virtualization group and enable libvirtd:

sudo dnf groupinstall -y "Virtualization"

sudo systemctl enable --now libvirtd

## 5.1 Host verification and gotchas (do these now)

Add your user to libvirt, then log out/in (or reboot) so virt-manager can manage VMs without constant auth prompts:

sudo usermod -aG libvirt $(whoami)

Confirm KVM is available and libvirt is healthy:

lsmod | egrep "kvm|kvm_amd"

sudo systemctl status libvirtd --no-pager

virt-host-validate

If virt-host-validate shows a FAIL related to virtualization being disabled, return to Section 3 and re-check SVM + IOMMU.

If virt-manager can’t connect, confirm the libvirtd service is running and that your user is in the libvirt group (run: groups).

# 6. Install NVIDIA driver for the host GTX 1080 (recommended)

Enable RPM Fusion and install akmod-nvidia, then reboot:

sudo dnf install -y https://mirrors.rpmfusion.org/free/fedora/rpmfusion-free-release-$(rpm -E %fedora).noarch.rpm https://mirrors.rpmfusion.org/nonfree/fedora/rpmfusion-nonfree-release-$(rpm -E %fedora).noarch.rpm

sudo dnf install -y akmod-nvidia xorg-x11-drv-nvidia

sudo reboot

# 7. VFIO passthrough: bind RTX 3090 to vfio-pci (host-side)

This step reserves the RTX 3090 for the Windows VM by binding it to vfio-pci.

## 7.1 Identify the RTX 3090 PCI IDs

Find the RTX 3090 and its HDMI/DP audio function IDs:

lspci -nn | egrep -i "vga|3d|audio"

Write down the two IDs for the RTX 3090, like: 10de:AAAA (GPU) and 10de:BBBB (Audio).

## 7.1A IOMMU group sanity check (don’t bind yet)

Before you bind anything to vfio-pci, confirm the RTX 3090 is in a sane IOMMU group.

Quick view (shows group number + device):

for g in /sys/kernel/iommu_groups/*; do

echo "IOMMU Group ${g##*/}"

for d in $g/devices/*; do lspci -nns ${d##*/}; done

done | less

What you want: the RTX 3090 (.0 VGA + .1 Audio) grouped together, and NOT bundled with unrelated controllers you need on the host.

If the group is messy: stop. Try the RTX 3090 in a different PCIe slot first. Only consider ACS override as a last resort (not baseline).

## 7.2 Add kernel parameters (GRUB)

Edit: /etc/default/grub

sudo nano /etc/default/grub

Append these to GRUB_CMDLINE_LINUX (keep existing options):

amd_iommu=on

iommu=pt

kvm.ignore_msrs=1

rd.driver.pre=vfio-pci

vfio-pci.ids=10de:AAAA,10de:BBBB   (use your real IDs)

video=efifb:off

Example:

GRUB_CMDLINE_LINUX="rhgb quiet amd_iommu=on iommu=pt kvm.ignore_msrs=1 rd.driver.pre=vfio-pci vfio-pci.ids=10de:AAAA,10de:BBBB video=efifb:off"

## 7.2A Fedora note (recommended on modern Fedora): use grubby instead of editing GRUB files

On many Fedora installs, kernel args are managed via BLS. If your GRUB edits don’t seem to apply, use grubby to set args persistently for ALL kernels:

sudo grubby --update-kernel=ALL --args="amd_iommu=on iommu=pt kvm.ignore_msrs=1 rd.driver.pre=vfio-pci vfio-pci.ids=10de:AAAA,10de:BBBB video=efifb:off"

To verify your kernel args:

sudo grubby --info=ALL | egrep -i "iommu=|vfio-pci.ids|rd.driver.pre"

If you use grubby, you can still keep the file-edit method above for reference, but grubby is the safer Fedora-native approach.

## 7.3 Regenerate GRUB config (UEFI)

sudo grub2-mkconfig -o /boot/efi/EFI/fedora/grub.cfg

## 7.4 Ensure VFIO loads early (dracut)

Create /etc/modprobe.d/vfio.conf:

sudo nano /etc/modprobe.d/vfio.conf

options vfio-pci ids=10de:AAAA,10de:BBBB

Rebuild initramfs and reboot:

sudo dracut -f

sudo reboot

## 7.5 Verify vfio-pci binding

After reboot, confirm the RTX 3090 is bound to vfio-pci:

lspci -k -nn | egrep -A3 -i "nvidia|10de"

You want the RTX 3090 entries to show: Kernel driver in use: vfio-pci.

# 8. Windows VM resource plan (your bare-metal illusion)

## 8.1 CPU (elastic by default)

Set Windows VM to 20 cores / 40 threads. This is a ceiling, not a reservation. Fedora still uses all CPU when the VM is idle. Do not use pinning or isolcpus for baseline.

VM vCPUs: 40

Topology: 1 socket / 20 cores / 2 threads

CPU model: host-passthrough

## 8.2 RAM (elastic via ballooning)

RAM is made elastic explicitly with virtio ballooning. This allows Windows to grow up to a maximum while Fedora remains healthy and responsive.

Max RAM: 80–96 GB (recommended)

Min/Initial RAM: 16–24 GB

Virtio balloon device: enabled

Important: do not statically allocate 120 GB to the VM on a 128 GB host. If you ever raise Max RAM toward 120 GB, keep a healthy minimum margin for Fedora and test carefully.

# 9. Create the Windows VM (virt-manager)

Open Virtual Machine Manager (virt-manager).

Create a new VM using your Windows 10 ISO (or your Winhance-custom ISO).

Choose UEFI (OVMF) firmware and Q35 machine type.

Set CPU model to host-passthrough and assign 40 vCPUs (20C/40T).

Set RAM with elastic settings (initial/min + max) and enable ballooning (details below).

Use a VirtIO disk bus and VirtIO network device.

Customize configuration before install.

## 9.A Click-by-click VM setup inside Fedora (beginner-friendly)

Goal: create a clean, predictable VM definition before you ever install Windows.

1) Open Virtual Machine Manager (virt-manager).

2) Click Create a new virtual machine.

3) Choose Local install media (ISO). Select your Windows ISO.

4) When asked, pick the closest OS type (Windows 10/11).

5) CPU + RAM: set your baseline values (you can refine later).

6) Disk: create a qcow2 disk image (recommended for snapshots).

7) IMPORTANT: check “Customize configuration before install”.

8) In the Overview panel:

Set Firmware to UEFI (OVMF).

Set Chipset/Machine Type to Q35.

9) In CPUs:

Set CPU model to host-passthrough.

10) In NIC/Network:

Set device model to virtio.

11) Add the VirtIO driver ISO:

Add Hardware → Storage → CD-ROM → attach virtio-win.iso as a second CD-ROM.

12) If you are installing Windows 11:

Add Hardware → TPM → Emulated → Version 2.0.

13) GPU passthrough timing:

Preferred baseline: install Windows first (using the default virtual display), then add RTX 3090 passthrough (Section 9.1) and move to the physical monitor.

If you insist on installing directly on the RTX 3090 monitor and you get a black screen, fall back to the baseline method above. Don’t troubleshoot blind.

## 9.1 Add RTX 3090 passthrough devices

Add Hardware → PCI Host Device → select RTX 3090 VGA function (…:… .0).

Add Hardware → PCI Host Device → select RTX 3090 Audio function (…:… .1).

## 9.2 Add VirtIO driver ISO

Add Hardware → Storage → CD-ROM and attach virtio-win.iso (as a second CD-ROM).

## 9.3 Keyboard and mouse (recommended options)

Best: pass through a whole USB controller to the VM (Add Hardware → PCI Host Device → USB controller).

Second-best: pass through a dedicated keyboard and mouse as USB Host Devices.

## 9.4 Elastic RAM settings in virt-manager (exact)

In the VM details panel:

Memory: set Current/Initial (min) to 16384–24576 MB (16–24 GB).

Memory: set Maximum to 81920–98304 MB (80–96 GB).

Add Hardware → Balloon Device → Virtio (enable it).

If your virt-manager UI only shows a single memory number, you can set max via XML (see Appendix F in this guide).

# 10. Install Windows

Start the VM. Windows Setup should appear on the display connected to RTX 3090.

If no disk is visible at install, click Load driver and select the VirtIO storage driver from the virtio-win ISO.

Finish Windows installation. Prefer a local account. Disable optional data collection prompts.

After first boot: install VirtIO guest tools from the VirtIO ISO.

Install NVIDIA driver in Windows for RTX 3090.

## 10.A First-boot checklist (do this before hardening)

Confirm the core plumbing works before you start removing things.

Device Manager has no unknown devices (after VirtIO + NVIDIA installs).

Windows can reboot twice in a row without drama.

You can shut down the VM and start it again from Fedora reliably.

If you are using the physical RTX 3090 monitor: confirm the monitor input is correct before assuming the VM is “black screen”.

# 11. Winhance and update strategy (Win10/Win11)

Use Winhance/custom ISO to debloat and reduce telemetry at install time.

Allow security updates; avoid feature upgrades.

Snapshot before patching and after major configuration changes.

## 11.1 Winhance: what it handles vs what you must still enforce

Winhance is great for initial cleanup, but Windows feature updates can reintroduce provisioned apps. The Steward approach is: debloat once, then enforce.

Rule: do not harden until you have Snapshot 1 (Section 12).

## 11.2 “Security updates only” without surprise feature upgrades

Goal: stay on your current feature release, take monthly quality/security updates, and only do feature upgrades when YOU decide.

On Pro/Enterprise/Education you can use Local Group Policy:

Computer Configuration → Administrative Templates → Windows Components → Windows Update → Windows Update for Business → “Select the target Feature Update version”.

Set it to your current Windows version (check by running: winver).

(If you prefer registry instead of Group Policy, apply the same policy under: HKLM\SOFTWARE\Policies\Microsoft\Windows\WindowsUpdate — TargetReleaseVersion, TargetReleaseVersionInfo, and ProductVersion.)

## 11.3 Deprovisioning UWP apps so they don’t come back for new users

Key idea: Remove-AppxProvisionedPackage prevents future installs for NEW user profiles. It does NOT remove the app from existing user profiles.

Step 1 — List provisioned packages (names you can target):

Get-AppxProvisionedPackage -Online | Format-Table DisplayName, PackageName

Step 2 — Deprovision a package (paste the full PackageName):

Remove-AppxProvisionedPackage -Online -PackageName "{PASTE_FULL_PACKAGE_NAME_HERE}"

Step 3 — Also remove the installed app for current users (so it’s gone NOW):

Get-AppxPackage -AllUsers "{APP_NAME_OR_WILDCARD}" | Remove-AppxPackage -AllUsers

Step 4 — Verify the deprovision registry key exists:

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\Appx\AppxAllUserStore\Deprovisioned\

## 11.4 Copilot + Recall (the two you called out)

Copilot (as an app): use the same two-step pattern — remove for existing users, then deprovision if it shows up as provisioned.

Example discovery:

Get-AppxPackage -AllUsers *Copilot* | Select Name, PackageFullName

Recall: treat it like an optional Windows feature. In Settings search: “Turn Windows features on or off” → uncheck Recall → restart.

If you want a PowerShell-safe approach, first discover the exact feature name on your build:

Get-WindowsOptionalFeature -Online | Where-Object FeatureName -match "Recall"

Then disable it using the discovered FeatureName (do not guess feature names).

## 11.5 Enforcement (so removals don’t quietly return)

If an app returns after an update, it means one of these is true:

It was removed only for the current user (not deprovisioned),

or a feature update reintroduced it and you need an enforcement step (scheduled re-removal) after updates.

Steward move: keep a tiny PowerShell “re-removal” script and run it after feature upgrades (or schedule it). See the added script in this package.

# 12. Snapshots, backups, and recovery discipline

Snapshot 1: Windows installed + VirtIO + NVIDIA driver working.

Snapshot 2: After Winhance/privacy hardening.

Snapshot 3: Before each monthly update cycle.

Back up VM disk file(s) and the VM XML definition regularly.

## 12.A How to snapshot (virt-manager) — exact clicks + naming

Snapshots are your seatbelt. Use them like a professional, not like a gambler.

In virt-manager:

Shut down the VM (don’t snapshot a moving target for baseline snapshots).

Open the VM → click the “Snapshots” button → Take Snapshot.

Name them plainly (examples):

SNAP_01_WIN_INSTALLED_DRIVERS_OK

SNAP_02_HARDENED_WINHANCE_DONE

SNAP_03_PRE_PATCH_2025-12

SNAP_04_POST_PATCH_2025-12_OK

## 12.B Backups (separate from snapshots)

Snapshots are not backups. Backups survive disk failure and operator error.

VM disk image (qcow2) location is typically under /var/lib/libvirt/images/ (unless you chose a different path).

Export the VM definition (XML):

virsh dumpxml "<VM_NAME>" > "<VM_NAME>.xml"

Copy both the qcow2 and the .xml to your backup drive when the VM is powered off.

# 13. Verification (don’t guess)

Run the verification script on the Fedora host:

chmod +x check_vfio_setup.sh

./check_vfio_setup.sh

## 13.A Extra verification commands (when you want to double-check)

Host virtualization health:

virt-host-validate

Confirm IOMMU is actually enabled:

dmesg | egrep -i "iommu|amd-vi|dmAR" | head -n 50

Confirm vfio modules loaded:

lsmod | egrep "vfio|vfio_pci|vfio_iommu_type1"

Confirm the RTX 3090 is bound to vfio-pci:

lspci -k | egrep -A3 -i "nvidia|10de:"

VM definition sanity (once created):

virsh list --all

virsh dominfo "<VM_NAME>"

# 14. Optional conservative enhancements (safe)

Barrier hotkey for single keyboard/mouse across host + VM (optional).

Looking Glass (optional) once everything is stable.

# Appendix F — Elastic RAM via VM XML (if needed)

If virt-manager does not expose min/max memory cleanly, you can set it in XML. Example pattern:

<memory unit='KiB'>98304000</memory>         <!-- max: 96 GB -->

<currentMemory unit='KiB'>24576000</currentMemory> <!-- current/min: 24 GB -->

Also ensure a virtio balloon device exists in XML.