Fedora + Windows VM Ultimate Steward Guide (v5 — Full)

TRX40 AORUS MASTER (rev 1.0) • Threadripper 3960X • GTX 1080 Host • RTX 3090 VM • Elastic CPU + Elastic RAM

# 0. What this system is

Fedora is your real operating system. Windows lives inside a VM and only runs when you start it. The RTX 3090 is passed through to Windows for near-native GPU performance. The GTX 1080 stays with Fedora so the host remains stable, responsive, and in control.

# 1. The Steward Philosophy (read once, then follow the steps)

Conservative first: build a baseline that always boots and always recovers.

Elastic CPU is default: Linux schedules CPU time dynamically; no pinning needed.

Elastic RAM is explicit: use virtio ballooning so Windows can grow while Fedora stays healthy.

No guesswork: change one thing at a time; snapshot before risk.

# 2. Target Hardware + Layout

CPU: AMD Threadripper 3960X (24C/48T)

RAM: 128 GB DDR4

Motherboard: Gigabyte TRX40 AORUS MASTER (rev 1.0)

Host GPU (Fedora): GTX 1080

VM GPU (Windows): RTX 3090

Recommended physical layout (simple and stable):

GTX 1080 in the top x16 slot (host display GPU).

RTX 3090 in the next full-length x16 slot (passthrough GPU).

Monitor 1 plugged into GTX 1080 (Fedora).

Monitor 2 or a second input plugged into RTX 3090 (Windows VM).

# 3. BIOS setup — exact menu paths (TRX40 AORUS MASTER)

Enter BIOS: press DEL during boot. Press F2 to switch to Advanced Mode. Change only these settings:

## 3.1 Enable CPU virtualization

Path: Advanced Mode → Tweaker → Advanced CPU Settings

SVM Mode → Enabled

## 3.2 Enable IOMMU

Path: Advanced Mode → Settings → IO Ports

IOMMU → Enabled

## 3.3 PCIe addressing and memory mapping

Path: Advanced Mode → Settings → IO Ports

Above 4G Decoding → Enabled

Re-Size BAR Support → Disabled (keep disabled for baseline stability)

## 3.4 Force host GPU as boot display

Path: Advanced Mode → Settings → IO Ports

Initial Display Output → the PCIe slot containing the GTX 1080

## 3.5 UEFI-only boot

Path: Advanced Mode → Boot

CSM Support → Disabled

## 3.6 Disable Secure Boot (baseline)

Path: Advanced Mode → Boot → Secure Boot

Secure Boot Enable → Disabled

## 3.7 Save and exit

Path: Advanced Mode → Save & Exit

Save & Exit Setup → Confirm, then reboot once before installing Fedora

# 4. Install Fedora (host OS)

Install Fedora Workstation (or Fedora KDE if you prefer a more traditional desktop).

During install: consider full-disk encryption if you want maximum privacy at rest.

After first boot, update the system and reboot.

sudo dnf upgrade -y

sudo reboot

# 5. Install virtualization stack (KVM/QEMU/libvirt/virt-manager)

Install the Fedora virtualization group and enable libvirtd:

sudo dnf groupinstall -y "Virtualization"

sudo systemctl enable --now libvirtd

# 6. Install NVIDIA driver for the host GTX 1080 (recommended)

Enable RPM Fusion and install akmod-nvidia, then reboot:

sudo dnf install -y https://mirrors.rpmfusion.org/free/fedora/rpmfusion-free-release-$(rpm -E %fedora).noarch.rpm https://mirrors.rpmfusion.org/nonfree/fedora/rpmfusion-nonfree-release-$(rpm -E %fedora).noarch.rpm

sudo dnf install -y akmod-nvidia xorg-x11-drv-nvidia

sudo reboot

# 7. VFIO passthrough: bind RTX 3090 to vfio-pci (host-side)

This step reserves the RTX 3090 for the Windows VM by binding it to vfio-pci.

## 7.1 Identify the RTX 3090 PCI IDs

Find the RTX 3090 and its HDMI/DP audio function IDs:

lspci -nn | egrep -i "vga|3d|audio"

Write down the two IDs for the RTX 3090, like: 10de:AAAA (GPU) and 10de:BBBB (Audio).

## 7.2 Add kernel parameters (GRUB)

Edit: /etc/default/grub

sudo nano /etc/default/grub

Append these to GRUB_CMDLINE_LINUX (keep existing options):

amd_iommu=on

iommu=pt

kvm.ignore_msrs=1

rd.driver.pre=vfio-pci

vfio-pci.ids=10de:AAAA,10de:BBBB   (use your real IDs)

video=efifb:off

Example:

GRUB_CMDLINE_LINUX="rhgb quiet amd_iommu=on iommu=pt kvm.ignore_msrs=1 rd.driver.pre=vfio-pci vfio-pci.ids=10de:AAAA,10de:BBBB video=efifb:off"

## 7.3 Regenerate GRUB config (UEFI)

sudo grub2-mkconfig -o /boot/efi/EFI/fedora/grub.cfg

## 7.4 Ensure VFIO loads early (dracut)

Create /etc/modprobe.d/vfio.conf:

sudo nano /etc/modprobe.d/vfio.conf

options vfio-pci ids=10de:AAAA,10de:BBBB

Rebuild initramfs and reboot:

sudo dracut -f

sudo reboot

## 7.5 Verify vfio-pci binding

After reboot, confirm the RTX 3090 is bound to vfio-pci:

lspci -k -nn | egrep -A3 -i "nvidia|10de"

You want the RTX 3090 entries to show: Kernel driver in use: vfio-pci.

# 8. Windows VM resource plan (your bare-metal illusion)

## 8.1 CPU (elastic by default)

Set Windows VM to 20 cores / 40 threads. This is a ceiling, not a reservation. Fedora still uses all CPU when the VM is idle. Do not use pinning or isolcpus for baseline.

VM vCPUs: 40

Topology: 1 socket / 20 cores / 2 threads

CPU model: host-passthrough

## 8.2 RAM (elastic via ballooning)

RAM is made elastic explicitly with virtio ballooning. This allows Windows to grow up to a maximum while Fedora remains healthy and responsive.

Max RAM: 80–96 GB (recommended)

Min/Initial RAM: 16–24 GB

Virtio balloon device: enabled

Important: do not statically allocate 120 GB to the VM on a 128 GB host. If you ever raise Max RAM toward 120 GB, keep a healthy minimum margin for Fedora and test carefully.

# 9. Create the Windows VM (virt-manager)

Open Virtual Machine Manager (virt-manager).

Create a new VM using your Windows 10 ISO (or your Winhance-custom ISO).

Choose UEFI (OVMF) firmware and Q35 machine type.

Set CPU model to host-passthrough and assign 40 vCPUs (20C/40T).

Set RAM with elastic settings (initial/min + max) and enable ballooning (details below).

Use a VirtIO disk bus and VirtIO network device.

Customize configuration before install.

## 9.1 Add RTX 3090 passthrough devices

Add Hardware → PCI Host Device → select RTX 3090 VGA function (…:… .0).

Add Hardware → PCI Host Device → select RTX 3090 Audio function (…:… .1).

## 9.2 Add VirtIO driver ISO

Add Hardware → Storage → CD-ROM and attach virtio-win.iso (as a second CD-ROM).

## 9.3 Keyboard and mouse (recommended options)

Best: pass through a whole USB controller to the VM (Add Hardware → PCI Host Device → USB controller).

Second-best: pass through a dedicated keyboard and mouse as USB Host Devices.

## 9.4 Elastic RAM settings in virt-manager (exact)

In the VM details panel:

Memory: set Current/Initial (min) to 16384–24576 MB (16–24 GB).

Memory: set Maximum to 81920–98304 MB (80–96 GB).

Add Hardware → Balloon Device → Virtio (enable it).

If your virt-manager UI only shows a single memory number, you can set max via XML (see Appendix F in this guide).

# 10. Install Windows

Start the VM. Windows Setup should appear on the display connected to RTX 3090.

If no disk is visible at install, click Load driver and select the VirtIO storage driver from the virtio-win ISO.

Finish Windows installation. Prefer a local account. Disable optional data collection prompts.

After first boot: install VirtIO guest tools from the VirtIO ISO.

Install NVIDIA driver in Windows for RTX 3090.

# 11. Winhance and update strategy (Win10/Win11)

Use Winhance/custom ISO to debloat and reduce telemetry at install time.

Allow security updates; avoid feature upgrades.

Snapshot before patching and after major configuration changes.

# 12. Snapshots, backups, and recovery discipline

Snapshot 1: Windows installed + VirtIO + NVIDIA driver working.

Snapshot 2: After Winhance/privacy hardening.

Snapshot 3: Before each monthly update cycle.

Back up VM disk file(s) and the VM XML definition regularly.

# 13. Verification (don’t guess)

Run the verification script on the Fedora host:

chmod +x check_vfio_setup.sh

./check_vfio_setup.sh

# 14. Optional conservative enhancements (safe)

Barrier hotkey for single keyboard/mouse across host + VM (optional).

Looking Glass (optional) once everything is stable.

# Appendix F — Elastic RAM via VM XML (if needed)

If virt-manager does not expose min/max memory cleanly, you can set it in XML. Example pattern:

<memory unit='KiB'>98304000</memory>         <!-- max: 96 GB -->

<currentMemory unit='KiB'>24576000</currentMemory> <!-- current/min: 24 GB -->

Also ensure a virtio balloon device exists in XML.